Q1:

LED Period = 2  (PSC + 1)  (ARR + 1) / f_CLK

Where:

PSC: Prescaler value (7199 in code)

ARR: Clock Counter / Auto-reload value (4999 in code)

f_CLK: Input clock frequency (usually 72,000,000 Hz)

2: Because the LED toggles (ON/OFF) each time the counter finishes, so one full cycle needs two toggles.

Q2:

in EIE3810_SYSTICK_Init(), modify “SysTick->LOAD = 72000 - 1” to “36000 - 1”.

Q3:

- If LED0_PWM_VAL is 0:

Oscilloscope Signal: Constantly HIGH.

LED0 Status: Fully ON (Maximum Brightness).

- If LED0_PWM_VAL is 9999:

Oscilloscope Signal: Almost constantly LOW (3.3V) (Duty cycle is approximately 100%).

LED0 Status: OFF (or extremely dim).

- If LED0_PWM_VAL is 10000:

If LED0_PWM_VAL is 10000:

Oscilloscope Signal: Constantly Low  (3.3V).

LED0 Status: Fully OFF.